Preparation of CaSO4:Dy by precipitation method to gamma

radiation dosimetry

T. Rivera a,_, J. Romana, J. Azorı´n b, R. Sosa b, J. Guzma´n a, A.K. Serrano a, M. Garcı´a c, G. Alarco´n a

a Centro de Investigacio´n en Ciencia Aplicada y Tecnologı´a Avanzada-IPN, Unidad Legaria, 11500 Me´xico D.F, Mexico

b Departamento de Fı´sica, Universidad Auto´noma Metropolitana-Iztapalapa, 09340 Me´xico D.F, Mexico

c Institutode Investigaciones en Materiales, UNAM. Circuito interior S/N. 04520 Me´xico D.F, Mexico

a r t i c l e i n f o

Keywords:

Thermoluminescence

Dy3+

Gamma radiation

CaSO4

Chemical co-precipitation

Dosimetry

a b s t r a c t

This paper presents the results of the preparation and characterization of dysprosium-doped calcium sulfate

(CaSO4:Dy) phosphor, which was obtained by homogeneous precipitation from calcium acetate

Ca(CH3COO_)2. Structural and morphological characteristics were studied using a scanning electronic

microscope (SEM). The structure of all compounds was determined by X-ray diffraction method too.

Thermoluminescence (TL) emission properties of CaSO4:Dy under gamma radiation effects were studied. This

phosphor powder presented a TL glow curve with two peaks (Tmax) centered at around of 180 and 300 1C,

respectively. The TL response of CaSO4:Dy as a function of gamma absorbed dose was linear in a wide range.

Both emission and excitation spectrawere also obtained. Results showed that this new preparation method of

CaSO4:Dy TL phosphor is less expensive, cleaner and safer than the conventional preparation method.