Journal of Alloys and Compounds 479 (2009) 511–519

Contents lists available at ScienceDirect

Journal of Alloys and Compounds

journal homepage: www.elsevier.com/locate/jallcom

Structural and morphology comparison between m-LaVO4 and LaVO3 compounds

prepared by sol–gel acrylamide polymerization and solid state reaction

G. Herreraa,∗, E. Chavirab, J. Jiménez-Mierc, A. Ordo˜nezb, E. Fregoso-Israel b, L. Ba˜nosb, E. Bucioc,

J. Guzmánb,d, O. Novelob, C. Floresb

a Posgrado en Ciencias Químicas, Universidad Nacional Autónoma de México, Circuito Exterior s/n, 70-360, 04510 México D.F., Mexico

b Instituto de Investigaciones en Materiales, Universidad Nacional Autónoma de México, 70-360, 04510 México D.F., Mexico

c Instituto de Ciencias Nucleares, Universidad Nacional Autónoma de México, 70-543, 04510 México D.F., Mexico

d Centro de Investigación en Ciencia Aplicada y Tecnología Avanzada, Instituto Politécnico Nacional 11500 México D.F., Mexico

a r t i c l e i n f o

Article history:

Received 28 October 2008

Received in revised form

22 December 2008

Accepted 23 December 2008

Available online 19 January 2009

Keywords:

Ceramics

Sol–gel processes

Microstructure

Atomic force microscopy (AFM)

X-ray diffraction

a b s t r a c t

We contrast the production of LaVO3 polycrystalline samples obtained by reduction of m-LaVO4 prepared

by sol–gel acrylamide polymerization (SGAP) and solid state reaction (SSR). For SGAP the formation of m-

LaVO4 occurs at 400 ◦C, for SSR at 1400 ◦C. For m-LaVO4-SGAP we observe a homogeneous morphology

with needle-shaped grains of 50nm average size. The SSR presents a broader size distribution in the

micrometer range. Both m-LaVO4 samples were reduced into LaVO3 using a Zr rod at 850 ◦C in vacuum.

LaVO3-SGAP presents a homogeneous grain distribution with an average size of 745 nm. LaVO3-SSR has

an average size of 3.45_m. The stoichiometry of all compoundswas confirmed by energy dispersive X-ray

spectroscopy. X-ray powder diffraction and transmission electron microscopy give crystal structures in

agreement with those reported in the literature.